# EXPERIMENT 1

# MINIMIZATION AND IMPLEMENTATION OF BOOLEAN FUNCTION

## Aim

To minimize and implement a Boolean function using logic gates and verify the functionality using Synopsys VCS and DVE.

## Software Required

- Synopsys VCS

- Synopsys DVE

- Linux Terminal

- Verilog HDL

## Boolean Function

Consider the Boolean function:

F(A,B,C,D)=Σm(0,2,5,7,8,10,13,15)

The minimized Boolean expression is: F=B'D'+BD

The function is implemented using Verilog HDL.

## Files Used

| File | Description |
| --- | --- |
| exp1_boolean_min.v | Verilog design file |
| exp1_boolean_min_tb.v | Verilog testbench |
| exp1_boolean_min.vcd | VCD waveform dump generated during simulation |

## 1. Verilog Design File

### //gedit exp1_boolean_min. v

module boolean(

input A,

input B,

input C,

input D,

output F

);

assign F = (~B & ~D) | (B & D);

endmodule

## 2. Testbench

### // gedit tb.v

module tb;

reg A;

reg B;

reg C;

reg D;

wire F;

exp1_boolean_min dut (

.A(A),

.B(B),

.C(C),

.D(D),

.F(F)

);

initial begin

$dumpfile("exp1_boolean_min.vcd");

$dumpvars(0, tb);

A=0; B=0; C=0; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=0; B=0; C=0; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=0; B=0; C=1; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=0; B=0; C=1; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=0; B=1; C=0; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=0; B=1; C=0; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=0; B=1; C=1; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=0; B=1; C=1; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=0; C=0; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=0; C=0; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=0; C=1; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=0; C=1; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=1; C=0; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=1; C=0; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=1; C=1; D=0; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

A=1; B=1; C=1; D=1; #10;

$display("A=%b B=%b C=%b D=%b F=%b",A,B,C,D,F);

$finish;

end

endmodule

## 3. Truth Table

| A | B | C | D | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

# 4. Simulation Procedure

## STEP 1 – Open Terminal

Open a terminal in the experiment folder.

bash

## STEP 2 – Load Synopsys Environment

source /synopsys/start.sh

## STEP 3 – Compile Using VCS

vcs exp1_boolean_min.v exp1_boolean_min_tb.v -full64

If compilation is successful, VCS generates the simulation executable:

simv

## STEP 4 – Run Simulation

./simv

The terminal displays the input combinations and corresponding output F.

A VCD waveform file is also generated:

exp1_boolean_min.vcd

## STEP 5 – Open DVE

dve -full64

### Other option

dve -full64 &

A DVE environment will open.

# 5. DVE Waveform Verification

In DVE:

- Open the testbench hierarchy.

- Locate the signals:

  - A

  - B

  - C

  - D

  - F

- Add the signals to the waveform window.

- Run/inspect the waveform.

- Verify that F = 1 whenever B and D are equal.

- Verify that F = 0 whenever B and D are different.

The waveform should agree with the truth table.

# 6. Expected Result

The Boolean function

F(A,B,C,D)=Σm(0,2,5,7,8,10,13,15), which simplifies to F=B'D'+BD.

was minimized to F=B'D'+BD.

and successfully implemented using Verilog HDL.

The design was compiled and simulated using Synopsys VCS, and the functionality was verified using DVE waveform analysis.

# 7. Viva-Voce Questions

- What is Boolean function minimization?

- What is a minterm?

- What is a Karnaugh map?

- Why is Boolean minimization required?

- What are universal gates?

- What is the difference between SOP and POS?

- What is the purpose of a Verilog testbench?

- Why is a VCD file generated?

- What is the purpose of ./simv?

- What is the purpose of DVE?

- What is the difference between simulation and synthesis?

- Why are A and C absent from the minimized expression?

- What is the purpose of $dumpfile?

- What is the purpose of $dumpvars?

## Result

Thus, the Boolean function was minimized, implemented using Verilog HDL, successfully simulated using Synopsys VCS, and verified using Synopsys DVE.